## LIST OF QUALITY-CONTROL SAMPLES

### List of quality-control sample Serial number; Name; Specifications; Test parameters. 1; Room temperature tensile test sample(Hot rolled ribbed steel bar); Ф12mm×350mm Ф16mm×350mm Ф16mm×450mm; Tensile strength (Rm, MPa) Lower yield strength (ReL, MPa) Percentage elongation after fracture (A, %). 2; Room temperature tensile test sample (Steel bar); Ф10mm, M16 standard thread Ф 5 mm, M12 standard thread; Tensile strength (Rm, MPa) Lower yield strength (ReL, MPa) Percentage elongation after fracture (A, %) Percentage reduction of area (Z, %). 3; Room temperature tensile test sample (Steel Plate); Thickness: 2 mm Width: 20 mm Original gage: 50 mm The parallel length: 90 mm Clamping head: 50 mm; Tensile strength (Rm, MPa) Lower yield strength (ReL, MPa) Percentage elongation after fracture (A, %). 4; Room temperature tensile test sample (Aluminium alloy Plate); Thickness: 2 mm Width: 20 mm Original gage: 50 mm The parallel length: 90 mm Clamping head: 50 mm; Tensile strength (Rm, MPa) Percentage elongation after fracture (A50mm, %) Proof strength, plastic extension of Rp0.2 (MPa). 5; Room temperature tensile test sample (Steel wire); Ф4mm×300mm; Tensile strength (Rm, MPa) Percentage elongation after fracture (A100mm, %). 6; Elevated temperature tensile test sample (Steel bar); Ф 5 mm, M12 standard thread; Tensile strength (Rm, MPa) Lower yield strength (ReL, MPa) Percentage elongation after fracture (A, %). 7; Charpy impact test sample (Metallic material); U5 notch, 10mmˣ10mmˣ55mm; Impact Absorbed Energy. 8; Brinell hardness test sample (Metallic material); 100mmˣ80mmˣ16mm 100mmˣ80mmˣ16mm 60mmˣ40mmˣ10mm; HBW 2.5/187.5 HBW 5/750 HBW 10/3000. 9; Rockwell hardness test sample (Metallic material); 60mmˣ40mmˣ10mm 60mmˣ40mmˣ10mm; HRB HRC. 10; Vickers hardness test sample (Metallic material); 25mmˣ25mmˣ5mm 60mmˣ40mmˣ10mm 60mmˣ40mmˣ10mm; HV0.2 HV5 HV10. 11; Neutral salt spray test sample (Metallic material); 50mm×50mm×2mm; The final quality loss (g/m2).. Contact Information: China NIL Research Center for Proficiency Testing Address: No.13, Gaoliangqiao Xiejie, Haidian District, Beijing, 100081, P.R.China Contacts: Ms. GUO Juan, Mr. Li Yinyan Fax: +86 10 6218 1163 Tel: +86 10 6218 1064/8330 E-mail:nil@analysis.org.cn